[image: image1.jpg]60% Dual Output Switching Power Supply D=60  seres

WOutiine and Dimension:

1525
T2
)
&
b
B
213
3o
S P
235
=
45

24.0 78.0




    

          乐清市乐宇自动化电气有限公司
                Yueqing Leyu Electric Automation Co., Ltd.


60W Dual Output Switching Power Supply    D-60 series

Features:

High efficiency, high reliability
Universal AC input /full range
100% full load burn-in test
Protections: Short circuit/ Over load/ Over voltage
LED indicator for power on
Cooling by free air convection
Fixed switching frequency at 54KHz
2 years warranty
	SPECIFICATIONS

	OUTPUT
	

	Model
	D-60A
	D-60B
	D-60C

	Output number
	CH1
	CH2
	CH1
	CH2
	CH1
	CH2

	DC Voltage 
	5V
	12V
	5V
	24V
	15V
	-15V

	Rated Current 
	4A
	3A
	2A
	2A
	2A
	1.5A

	Current Range 
	0.8-4A
	0.3-3A
	0.8-3A
	0.2-2A
	0.5-2A
	0.2-1.5A

	Rated Power 
	56W
	58W
	60W

	Ripple & Noise 
	80mVp-p
	100mVp-p
	80mVp-p
	120mVp-p
	100mVp-p
	120mVp-p

	Voltage Adj. Range
	CH1: -5%,+10%
	CH1: -5%,+10%
	CH1: -5%,+10%

	Voltage Tolerance 
	±2%
	±6%
	±3%
	±5%
	±3%
	±5%

	Setup,Rise,Hold Up Time 
	800ms,50ms,10ms/115VAC    300ms,50ms,80ms/230VAC at full load

	
	

	INPUT
	

	Voltage Range 
	85~132VAC/170~264VAC selected by switch   47~63Hz; 240~370VDC

	AC Current
	1.8A/115V   0.9A/230V

	Efficiency
	74%
	76%
	78%

	Inrush Current 
	Cold start 25A/115V   50A/230V

	Leakage Current
	<1mA/240VAC

	
	

	PROTECTION
	

	Over Load 
	105%~150% rated output power

	
	Protection type: Shut down o/p voltage, recovers automatically after fault condition is removed

	Over Voltage 
	CH1: 115%~135%

	
	Protection type: hiccup mode, auto-recovery after fault condition is removed

	
	

	ENVIRONMENT
	

	Working Temp., Humidity
	-10℃~+60℃; 20%~90%RH

	Storage Temp., Humidity
	-20℃~+85℃; 10%~95%RH non-condensing

	Temp. Coefficient
	±0.03%/℃(0~50℃)

	Vibration
	10~500Hz, 2G 10min./1cycle, period for 60min, each along X, Y, Z axes

	
	

	SAFETY
	

	Withstand Voltage
	I/P-O/P: 1.5KVAC   I/P-FG: 1.5KVAC   O/P-FG: 0.5KVAC

	Isolation Resistance
	I/P-O/P, I/P-FG, O/P-FG: 100M Ohms/500VDC

	
	

	STANDARD
	

	Safety Standard
	Design refer to UL1012, UL60950, EN60950

	EMC Standard
	Design refer to EN55022(CLSPR22),EN61000-3-2,-3, EN61000-4-2,3,4,5,6,8,11; ENV50204, EN55024

	
	

	OTHERS
	

	Dimension
	159*97*38mm(L*W*H)

	Weight
	0.55Kg

	Packing
	45pcs/25.5Kg/1.45CUFT

	
	

	NOTE
	

	1.All parameters NOT specially mentioned are measured at 230VAC input, rated load and 25℃ of ambient temperature.

2.Ripple & noise are measured at 20MHz of bandwidth by using a 12” twisted pair-wire terminated with a 0.1μf & 47μf parallel capacitor. 

3.Tolerance: includes set up tolerance, line regulation and load regulation.

4.The power supply is considered a component which will be installed into a final equipment. The final equipment must be re-confirmed that it still meets EMC directives.


[image: image3.jpg]LEYU! ©

S



[image: image2.jpg]159.0

220 117.0
n| [ FEE
| | 3-M3 2F
oo o =
g 2+ ™
1500
Terminal Pin No. Assignment:
ot [Fesgrmen] Pt | Aesgen
TR I O T
T Tun |5 Jewmrn
T e o7 [wamio
M Derating Curve W Static Characteristics (B)
1t nease
feT
L £ 2450 %
£ 8 e s
B 5 i
T ¥ § 244 4
o
: o 8
AT w W W w e (orzona W T T T T W 9 H0 60 60 (HORZONTAL)
: 58 Gamon e

AVBIENT TEMPERATURE (T) NPUTVOLTAGE (1) iz





